微算機系統

小組專案報告

實驗四

組別： 18

班級、姓名與學號：

資工二 蕭耕宏 110590005

資工二 楊榮鈞 110590034

日期： 2022.10.25

- 實驗內容：

利用component、package、port map、for generate、if generate完成7bits的ALU(功能包括and、or、add、subtract、set-less-than、nor)，並用七段顯示器顯示ALU的輸出結果，overflow用LED顯示。

- 實驗過程及結果：

全加器的真值表

根據上方真值表使用卡諾圖來化簡並設計出電路

卡諾圖化簡後設計出的電路

實驗的結果

進階題

| 111 1111(A, 右) and 111 1111(B, 左) = 111 1111 | 011 1111(A, 右) and 111 1110(B, 左) = 011 1110 |
| --- | --- |
| 101 0101(A, 右) or 010 1010(B, 左) = 111 1111 | 000 0011 (A, 右) or 011 0000(B, 左) = 011 0011 |
| 000 1000(A, 右) + 000 0100(B, 左) = 000 1100 (no overflow) | 100 0000(A, 右) + 100 0000(B, 左) = 000 0000 (overflow) |
| 000 1000(A, 右) - 000 0010(B, 左) = 000 0110 (no overflow) | 100 0000(A, 右) - 000 0001(B, 左) = 011 1111 (overflow) |
| 000 1000(A, 右) - 000 0100(B, 左) = 000 0100 > 0 (no overflow) Result = 000 0000 | 100 0000(A, 右) - 000 0001(B, 左) = 011 1111 > 0 (overflow) Result = 000 0000 |
| 001 0000(A, 右) - 010 0000(B, 左) = 111 0000 < 0 (no overflow) Result = 000 0001 | 010 0000(A, 右) - 100 0000(B, 左) = 110 0000 < 0 (overflow) Result = 000 0001 |
| 000 0001(A, 右) nor 000 0010(B, 左) = 111 1100 | 100 0000(A, 右) nor 100 0000(B, 左) = 011 1111 |

- 程式碼

進階題

fulladd.vhd

library ieee; use ieee.std_logic_1164.all; entity fulladd is port( Cin,x,y : in std_logic; s,Cout : out std_logic ); end fulladd; architecture func of fulladd is begin s <= x xor y xor Cin; Cout <= (x and y) or (Cin and x) or (Cin and y); end func;

hex.vhd

library ieee; use ieee.std_logic_1164.all; entity hex is port( SW2:in std_logic_vector(3 downto 0); HEX2:out std_logic_vector(6 downto 0); SW1:in std_logic_vector(3 downto 0); HEX1:out std_logic_vector(6 downto 0) ); end hex; architecture behavioral of hex is --boolean begin --a HEX2(0) <= (SW2(2) and not(SW2(1)) and not(SW2(0))) or (SW2(3) and SW2(2) and not(SW2(1))) or (not(SW2(3)) and not(SW2(2)) and not(SW2(1)) and SW2(0)) or (SW2(3) and not(SW2(2)) and SW2(1) and SW2(0)); --b HEX2(1) <= (SW2(3) and SW2(1) and SW2(0)) or (SW2(2) and SW2(1) and not(SW2(0))) or (SW2(3) and SW2(2) and not(SW2(0))) or (not(SW2(3)) and SW2(2) and not(SW2(1)) and SW2(0)); --c HEX2(2) <= (SW2(3) and SW2(2) and not(SW2(0))) or (SW2(3) and SW2(2) and SW2(1)) or (not(SW2(3)) and not(SW2(2)) and SW2(1) and not(SW2(0))); --d HEX2(3) <= (not(SW2(2)) and not(SW2(1)) and SW2(0)) or (SW2(2) and SW2(1) and SW2(0)) or (not(SW2(3)) and SW2(2) and not(SW2(1)) and not(SW2(0))) or (SW2(3) and not(SW2(2)) and SW2(1) and not(SW2(0))); --e HEX2(4) <= (not(SW2(3)) and SW2(0)) or (not(SW2(3)) and SW2(2) and not(SW2(1))) or (not(SW2(2)) and not(SW2(1)) and SW2(0)); --f HEX2(5) <= (SW2(3) and SW2(2) and not(SW2(1))) or (not(SW2(3)) and not(SW2(2)) and SW2(0)) or (not(SW2(3)) and not(SW2(2)) and SW2(1)) or (not(SW2(3)) and SW2(1) and SW2(0)); --g HEX2(6) <= (not(SW2(3)) and not(SW2(2)) and not(SW2(1))) or (not(SW2(3)) and SW2(2) and SW2(1) and SW2(0)); ----1 --a HEX1(0) <= (SW1(2) and not(SW1(1)) and not(SW1(0))) or (SW1(3) and SW1(2) and not(SW1(1))) or (not(SW1(3)) and not(SW1(2)) and not(SW1(1)) and SW1(0)) or (SW1(3) and not(SW1(2)) and SW1(1) and SW1(0)); --b HEX1(1) <= (SW1(3) and SW1(1) and SW1(0)) or (SW1(2) and SW1(1) and not(SW1(0))) or (SW1(3) and SW1(2) and not(SW1(0))) or (not(SW1(3)) and SW1(2) and not(SW1(1)) and SW1(0)); --c HEX1(2) <= (SW1(3) and SW1(2) and not(SW1(0))) or (SW1(3) and SW1(2) and SW1(1)) or (not(SW1(3)) and not(SW1(2)) and SW1(1) and not(SW1(0))); --d HEX1(3) <= (not(SW1(2)) and not(SW1(1)) and SW1(0)) or (SW1(2) and SW1(1) and SW1(0)) or (not(SW1(3)) and SW1(2) and not(SW1(1)) and not(SW1(0))) or (SW1(3) and not(SW1(2)) and SW1(1) and not(SW1(0))); --e HEX1(4) <= (not(SW1(3)) and SW1(0)) or (not(SW1(3)) and SW1(2) and not(SW1(1))) or (not(SW1(2)) and not(SW1(1)) and SW1(0)); --f HEX1(5) <= (SW1(3) and SW1(2) and not(SW1(1))) or (not(SW1(3)) and not(SW1(2)) and SW1(0)) or (not(SW1(3)) and not(SW1(2)) and SW1(1)) or (not(SW1(3)) and SW1(1) and SW1(0)); --g HEX1(6) <= (not(SW1(3)) and not(SW1(2)) and not(SW1(1))) or (not(SW1(3)) and SW1(2) and SW1(1) and SW1(0)); end behavioral;

mux_2to1.vhd

library ieee; use ieee.std_logic_1164.all; entity mux_2to1 is port( w0,w1 : in std_logic; s : in std_logic; f : out std_logic ); end mux_2to1; architecture behavioral of mux_2to1 is signal temp : std_logic; begin with s select f <= w0 when '0', w1 when others; end behavioral;

mux_4to1.vhd

library ieee; use ieee.std_logic_1164.all; entity mux_4to1 is port( w0,w1,w2,w3 : in std_logic; s : in std_logic_vector(1 downto 0); f : out std_logic ); end mux_4to1; architecture behavioral of mux_4to1 is begin with s select f <= w0 when "00", w1 when "01", w2 when "10", w3 when others; end behavioral;

onebitALU.vhd

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use work.lab4_package.all; entity onebitALU is port( A,B,less,carryin : in std_logic; opcode : in std_logic_vector(3 downto 0);--0000 result,set,carryout : out std_logic ); end onebitALU; architecture LogicFunc of onebitALU is signal Ain : std_logic; signal Bin : std_logic; signal w0_and: std_logic; signal w1_or : std_logic; signal w2_add: std_logic; -- set begin stage0:mux_2to1 port map(A,not A,opcode(3),Ain); stage1:mux_2to1 port map(B,not B,opcode(2),Bin); w0_and <= Ain and Bin; w1_or <= Ain or Bin; stage2:fulladd port map(Cin=>carryin,x=>Ain,y=>Bin,s=>w2_add,Cout=>carryout); set <= w2_add; stage3:mux_4to1 port map(w0=>w0_and,w1=>w1_or,w2=>w2_add,w3=>less,s(1 downto 0)=>opcode(1 downto 0),f=>result); end LogicFunc;

lab4_package.vhd

library ieee; use ieee.std_logic_1164.all; package lab4_package is component fulladd port( Cin,x,y : IN STD_LOGIC; s,Cout : OUT STD_LOGIC ); end component fulladd; component mux_4to1 port( w0,w1,w2,w3 : in std_logic; s : in std_logic_vector(1 downto 0); f : out std_logic ); end component mux_4to1; component mux_2to1 port( w0,w1 : in std_logic; s : in std_logic; f : out std_logic ); end component mux_2to1; component hex port( SW2:in std_logic_vector(3 downto 0); HEX2:out std_logic_vector(6 downto 0); SW1:in std_logic_vector(3 downto 0); HEX1:out std_logic_vector(6 downto 0) ); end component hex; component onebitALU port( A,B,less,carryin : in std_logic; opcode : in std_logic_vector(3 downto 0); result,set,carryout : out std_logic ); end component onebitALU; end lab4_package;

Lab4_1.vhd

library ieee; use ieee.std_logic_1164.all; use ieee.std_logic_unsigned.all; use work.lab4_package.all; entity Lab4_1 is port( A,B : in std_logic_vector(6 downto 0); opcode : in std_logic_vector(3 downto 0); overflow : out std_logic; HEX2:out std_logic_vector(6 downto 0); HEX1:out std_logic_vector(6 downto 0) ); end Lab4_1; architecture behavioral of Lab4_1 is signal result : std_logic_vector(7 downto 0); signal set : std_logic_vector(6 downto 0); signal carryout : std_logic_vector(6 downto 0); signal temp : std_logic_vector(9 downto 0); begin temp(9) <= opcode(2) and opcode(1); G1:for i in 0 to 6 generate G2:if i=0 generate stage0: onebitALU port map (A=>A(0),B=>B(0),less=>'0',carryin=>temp(9), opcode(3 downto 0)=>opcode(3 downto 0), result=>temp(0),set=>set(0),carryout=>carryout(0)); end generate; G3:if (i<6) and (i>0)generate stage1: onebitALU port map (A=>A(i),B=>B(i),less=>'0',carryin=>carryout(i-1), opcode(3 downto 0)=>opcode(3 downto 0), result=>result(i),set=>set(i),carryout=>carryout(i)); end generate; G4:if i=6 generate stage2: onebitALU port map (A=>A(i),B=>B(i),less=>'0',carryin=>carryout(i-1), opcode(3 downto 0)=>opcode(3 downto 0), result=>result(i),set=>set(i),carryout=>carryout(i)); stage3: onebitALU port map (A=>A(0),B=>B(0),less=>set(6),carryin=>temp(9), opcode(3 downto 0)=>opcode(3 downto 0), result=>result(0),set=>temp(1),carryout=>temp(2)); end generate; end generate; result(7) <= '0'; overflow <= carryout(6) xor carryout(5); stage4: hex port map (SW2(3 downto 0)=>result(7 downto 4),SW1(3 downto 0)=>result(3 downto 0), HEX2(6 downto 0)=>HEX2(6 downto 0), HEX1(6 downto 0)=>HEX1(6 downto 0)); end behavioral;

- 本次實驗過程說明與解決方法:

實驗過程:

基本題是實作一個1bit的ALU。

做法是使用兩個2對1的多工器決定A、B是否為not A、not B，然後再用2對1多工器得出的A、B去實作and、or、add (1bit的全加器)、subtract(1bit的全加器)、set-less-than (if(A-B)<0 then result = 1 else result = 0(用subtract去看))、nor(其做法為(not A) and (not B) = A nor B)，最後再用4對1多工器去選擇result要輸出什麼。

其中opcode有4bits，opcode[3]為Ainvert(用MUX去決定A還是not A)，opcode[2]為Bnegate(用MUX去決定B還是not B)，opcode[1]、opcode[0]為operation(用MUX去決定要輸出and、or、add、less哪一個)。

最後再用七段顯示器顯示出ALU的result，overflow則是以carrryin xor carryout決定是否亮LED燈。

在做上述動作時，分別要把1bit的全加器、1bit的ALU(overflow由主程式判斷，未放在ALU的package裡面)、七段顯示器給包裝成package。

而進階題就是直接利用基礎題的程式碼，做出7bits的ALU，再用七段顯示器輸出ALU的result，並且LSB的set-less-than的輸出是MSB(sign bit)的set的值，其餘的bit的set-less-than的輸出都是0。其中overflow一樣在主程式判斷，未放在1bit ALU的package中。

實作過程中有遇到一些小問題，就是我們對quartus的if generate的做法不熟悉，所以在寫MUX的時候用if generate沒寫出來，然後用老師的MUX的code的select with when的寫法的時候沒跑動。

解決方法:

1. if generate雖然是if的寫法，但是他不能把要輸入的東西當作判斷式，所以我們改用select with when的寫法，完成MUX的code。

2. 老師的MUX的code的select with when的寫法的時候沒跑動的原因是我們是先寫完老師的code再去試if generate能否寫出來的時候，我們在處理註解的時候不小心多加了一個符號在code裡面沒有發現所以沒跑動。在我們刪掉那個符號之後，MUX的code就能跑了。